RADON IN NATURAL GAS IN NEW YORK CITY

Radon

- Radon is a tasteless, odorless, colorless, naturally occurring radioactive noble gas created by the radioactive decay of uranium, thorium and radium.  It does not react with other elements.  It does not burn and can only be destroyed by radioactive decay.

- Radon (222Rn) has a half-life of 3.8 days.  It decays by alpha and beta particle emission until it becomes the stable isotope of lead (206Pb).  Two decay products are radioactive solids, polonium (210, 214, 218Po) and lead (210, 214Pb).  Polonium decays by alpha particle emission; lead decays by beta particle emission to create the radioactive Polonium isotopes.

- Radon and its radioactive decay products enter the body primarily through inhalation. Most of the radon is exhaled prior to radioactive decay while most of the solid radioactive polonium and lead remains in the lungs and will further decay to cause cancer.

- Radon is the leading cause of lung cancer among non-smokers and the second leading cause among smokers.  EPA attributes 21,000 lung cancer deaths per year on a nationwide basis to radon exposure.

- Although zero is the only “safe” level for radiation exposure, EPA urges mitigation for anyone with indoor levels at 4 picoCuries per liter and recommends mitigation beginning at 2 picoCuries per liter.

- Radon is present in all natural gas from deep shale formations but levels vary considerably from one field to another.  Available studies report that dark shales (eg. Marcellus) are more radioactive.

Current Radon Exposure Levels in NYC Metro Area

- Most natural gas consumed now in NYC metro area is from the Texas-Louisiana Gulf Coast and has a radon level at the well head of about 5 - 10 pCi/l according to published studies.

- Transit time to NYC metro area is 6 to 8 days.

- Average radon levels in gas currently delivered to NYC metro area apartments should be 1 to 2 pCi/l or less.

Future Radon Exposure Levels in NYC Metro Area with Marcellus Shale Gas

- Existing studies show Marcellus shale gas average radon level at well head is 150 to 160 pCi/l.

- Transit time to NYC metro area is approximately 15 to 20 hours depending on location of well, pipeline route, and transmission speed.

- Average radon level in Marcellus gas delivered to NYC metro area apartments will be about 125 pCi/l.

- Exposures to the elevated radioactivity levels in Marcellus gas will result in thousands of additional lung cancer deaths over time in NYC metro area.

Options to Protect Public Health

- Continue to use Gulf Coast gas; do not allow use of Marcellus gas.

- Store Marcellus gas long enough to decay to much less than 4 pCi/l before distribution.  Adding the required type of storage would be expensive.

- Refrigerate Marcellus gas to liquefy radon without liquefying the methane. The liquids will continue to be radioactive. This option would be expensive.

Jeff Zimmerman (jjzimmerman@comcast.net) or (240) 912-6685 (office) or (202) 262-9664 (cell)

for Damascus Citizens for Sustainability, Inc. and NYH2O, Inc.